S14-D2 
Evaluation of ET using AHP

Figure 1: Matrix of evaluation criteria.

# The group will compare environmental technologies through AHP (Analytic Hierarchy Process), methodology by filling in the table in figure above, and using the scale of AHP values represented in the table below.

- Matrix of evaluation criteria: criteria will be compared for couples, according to the scale below attached.

- Matrix of priorities: both TA are compared according to each criterion

- Final matrix: geometrically averaging the individual trials and the excel worksheet will be filled

| Scale AHP for comparison of pairs |  |
| --- | --- |
| 1 | Equal importance |
| 3 | Moderate importance one over the other |
| 5 | Strong or demonstrated importance |
| 7 | Strong or demonstrated importance |
| 9 | Extreme importance |
| 2, 4, 6, 8 | Intermediate values |

Figure 2: AHP scale for comparison of pairs

NOTES FOR TRAINERS

TASKS

# The goal is to become familiar with a complex multi criteria evaluation method. The AHP (Analytic Hierarchy Process) method is based on comparisons of pairs of elements, allowing integrating the views of a multidisciplinary team, and intuitive tool which favours rapid and objective results. # AHP procedure is as follows: # 1 Objective is determined to follow, in our case a measure environmental improvement. # 2. You define criteria, for example, investment costs, maintenance costs, effects on water, on air, on public health, enterprise competitiveness, difficulty in use, energy efficiency, consumption of resources, etc. # 3. Are taken into account the available ET for the objective to be achieved (e.g. for manufacturing process with contamination of sewage filtration and chlorination). # After the evaluation criteria and the possible ET are defined, the participants relate all criteria with all the ET (Figure 1) by assigning a value to every relationship using the AHP (Figure 2) scale for comparison of pairs. It means that criterion or measure that for oneself is more important or relevant notice with a higher score on the position of the matrix either 7 or a 1/7 e.g., depending on whether the left or above.

TIPS FOR CONDUCTING

# We recommend starting citing an example for a better understanding of the assessment process. # The end results of groups or participants separately are joined and it is passed to the automatic calculation of results measurement exercise S14-D3 .these highest are the indicative of choice.

FRAMEWORK

Number of participants: arbitrary Time requirement (approximately): 15 min Material: Cardstock, calculator, computer and presentation material

www.eurocrafts21.EU